# 1) Component List

Mandatory Components

- ESP32 Dev Board

- Ultrasonic Sensor (HC-SR04)

- Humidifier / Mist Module (with digital control pin)

- 5V power for ESP32

- Voltage Divider for ECHO pin

- 1 kΩ resistor

- 2 kΩ resistor

- Jumper wires

# 2) Connections

## Ultrasonic Sensor HC-SR04

| HC-SR04 Pin | ESP32 Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | GPIO 27 |
| ECHO | GPIO 26 |

## Humidifier Module

| Humidifier Pin | ESP32 Pin |
| --- | --- |
| SIGNAL | GPIO 25 |
| GND | GND |
| VCC | 5v |

3) Code

#define TRIG 27

#define ECHO 26

#define HUMIDIFIER 25

unsigned long lastPing = 0;

// states for humidifier sequence

bool sequenceRunning = false;

unsigned long seqTimer = 0;

int seqStep = 0;

void setup() {

Serial.begin(115200);

pinMode(TRIG, OUTPUT);

pinMode(ECHO, INPUT);

pinMode(HUMIDIFIER, OUTPUT);

digitalWrite(HUMIDIFIER, HIGH);  // idle = HIGH

}

// FAST ultrasonic

int getDistanceFast() {

long duration;

digitalWrite(TRIG, LOW);

delayMicroseconds(2);

digitalWrite(TRIG, HIGH);

delayMicroseconds(8);

digitalWrite(TRIG, LOW);

duration = pulseIn(ECHO, HIGH, 20000);

if (duration == 0) return -1;

return duration * 0.034 / 2;

}

// -------------------------

// HUMIDIFIER SEQUENCE

// -------------------------

void triggerHumidifierSequence() {

if (sequenceRunning) return;

sequenceRunning = true;

seqStep = 1;

seqTimer = millis();

}

// non-blocking sequence handler

void updateHumidifierSequence() {

if (!sequenceRunning) return;

unsigned long now = millis();

switch (seqStep) {

case 1: // FIRST SIGNAL (ON)

digitalWrite(HUMIDIFIER, LOW);

if (now - seqTimer >= 100) {   // 0.1 sec

digitalWrite(HUMIDIFIER, HIGH);

seqStep = 2;

seqTimer = now;

}

break;

case 2: // WAIT 0.9 sec before second signal

if (now - seqTimer >= 900) {

digitalWrite(HUMIDIFIER, LOW);

seqStep = 3;

seqTimer = now;

}

break;

case 3: // SECOND SIGNAL (closing soon)

if (now - seqTimer >= 100) {

digitalWrite(HUMIDIFIER, HIGH);

seqStep = 4;

seqTimer = now;

}

break;

case 4: // WAIT 0.1 sec before final close pulse

if (now - seqTimer >= 100) {

digitalWrite(HUMIDIFIER, LOW);

seqStep = 5;

seqTimer = now;

}

break;

case 5: // THIRD SIGNAL (closed)

if (now - seqTimer >= 100) {

digitalWrite(HUMIDIFIER, HIGH);

sequenceRunning = false; // done

}

break;

}

}

void loop() {

// ultrasonic ping every 15ms

if (millis() - lastPing >= 15) {

lastPing = millis();

int distance = getDistanceFast();

Serial.println(distance);

// Start sequence only if bee detected

if (distance > 0 && distance < 20 && !sequenceRunning) {

triggerHumidifierSequence();

}

}

// update humidifier state machine

updateHumidifierSequence();

}

## Changes for Mist Timing.

Now its set to 1 sec 900 mean 0.9 sec but 0.1 sec is closing time so if you want to increase then 
1900 for 2 sec 
2900 foe 3 sec
3900 for 4 sec ans soo on